1. Linh kiện

Arduino (chính) + Esp32

Mạch điều khiển động cơ L298N

Cảm biến chuyển động PIR

Màn hình LCD I2C

Module RFID RC522

Còi báo

Động cơ DC

Công tắc hành trình

Pin Lithium 3,7V

Server

2. Hệ thống

RFID, PIR, L298N, Còi, LCD, Công tắc hành trình

Web/App

Stm32

Esp32

Sơ đồ gồm 3 tầng, mỗi tầng chỉ được giao tiếp với tầng kế tiếp hoặc dưới nó:

- Server:

- Nhận thông tin từ ESP32

- Kiểm tra mã từ thẻ thông qua truy vấn cơ sở dữ liệu

- Truyền thông tin xuống các tầng dưới (cho phép mở cửa hoặc không)

- ESP32: Giúp Arduino kết nối với mạng, giao tiếp với Socket server

- Web/App:

Admin:

- - Đăng nhập

- - Đăng ký tài khoản cho người dùng

- - Mở/Đóng cửa

- - Quản lý người dùng + thẻ(Xem ds/Thêm/Sửa/Xóa)

- - Xem trạng thái cửa (đã mở, đang mở, đã đóng, đang đóng)

- - Xem lịch sử (tên, thời gian, mở bằng gì(thẻ/web/app))

- - Cho phép quét thẻ/Khóa hệ thống (khi khóa thì không thể mở cửa)

- - Thông báo: có người mở cửa, quét thẻ sai, có chuyển động (PIR)

Người dùng:

- - Đăng nhập

- - Quét thẻ/Dùng web/app

- - Xem trạng thái cửa

- Arduino:

- Tiếp nhận và gửi thông tin lên Server qua ESP32

- Lấy thông tin và điều khiển các module RFID, PIR, L298N, LCD, Còi, Công tắc hành trình

Code STM32:

#include <Arduino.h>

#include <STM32FreeRTOS.h>

#include <SPI.h>

#include <MFRC522.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// ================= PIN DEFINITIONS =================

#define SS_PIN PA4

#define RST_PIN PA3

#define MOTOR_IN1 PB0

#define MOTOR_IN2 PB1

#define MOTOR_ENA PA8

#define PIR_PIN PA0

#define BUZZER_PIN PA1

#define LIMIT_OPEN_PIN PC13

#define LIMIT_CLOSE_PIN PC14

// ================= UART =================

HardwareSerial Serial1(PA10, PA9);  // UART1: RX, TX to ESP32

HardwareSerial Serial3(PB11, PB10); // UART3: RX, TX for debug

// ================= OBJECTS =================

MFRC522 rfid(SS_PIN, RST_PIN);

LiquidCrystal_I2C lcd(0x27, 16, 2);

// ================= RTOS HANDLES =================

SemaphoreHandle_t xMutexSerial;

SemaphoreHandle_t xMutexLCD;

QueueHandle_t xQueueMotor;

// ================= STATE VARIABLES =================

volatile bool doorOpen = false;

volatile bool motorRunning = false;

volatile bool systemReady = false;

volatile bool autoCloseScheduled = false;

volatile TickType_t doorOpenedTime = 0;

String lastUserName = "";

// ================= MOTOR COMMANDS =================

enum MotorCmd

{

CMD_OPEN,

CMD_CLOSE,

CMD_STOP

};

// ================= HELPER FUNCTIONS =================

void sendToEsp(const char *msg)

{

if (xSemaphoreTake(xMutexSerial, pdMS_TO_TICKS(100)) == pdTRUE)

{

Serial1.println(msg);

Serial.println(msg);

Serial3.println(msg);

xSemaphoreGive(xMutexSerial);

}

}

void lcdShow(const char *l1, const char *l2 = "")

{

if (xSemaphoreTake(xMutexLCD, pdMS_TO_TICKS(100)) == pdTRUE)

{

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(l1);

if (strlen(l2) > 0)

{

lcd.setCursor(0, 1);

lcd.print(l2);

}

Serial3.print("[LCD] ");

Serial3.print(l1);

if (strlen(l2) > 0)

{

Serial3.print(" | ");

Serial3.println(l2);

}

else

{

Serial3.println();

}

xSemaphoreGive(xMutexLCD);

}

}

void buzzSuccess()

{

digitalWrite(BUZZER_PIN, HIGH);

vTaskDelay(pdMS_TO_TICKS(200));

digitalWrite(BUZZER_PIN, LOW);

}

void buzzError()

{

for (int i = 0; i < 3; i++)

{

digitalWrite(BUZZER_PIN, HIGH);

vTaskDelay(pdMS_TO_TICKS(150));

digitalWrite(BUZZER_PIN, LOW);

vTaskDelay(pdMS_TO_TICKS(150));

}

}

void closeDoor()

{

MotorCmd cmd = CMD_CLOSE;

xQueueSend(xQueueMotor, &cmd, 0);

}

void openDoor()

{

MotorCmd cmd = CMD_OPEN;

xQueueSend(xQueueMotor, &cmd, 0);

}

// ================= TASK: STARTUP =================

void taskStartup(void *pv)

{

(void)pv;

lcdShow("Xin chao!");

vTaskDelay(pdMS_TO_TICKS(3500));

lcdShow("Cua dang dong...");

closeDoor();

vTaskDelay(pdMS_TO_TICKS(1500));

lcdShow("Moi quet the...");

systemReady = true;

vTaskDelete(NULL);

}

// ================= TASK: RFID READER =================

void taskRFID(void *pv)

{

(void)pv;

while (1)

{

if (!systemReady)

{

vTaskDelay(pdMS_TO_TICKS(100));

continue;

}

if (rfid.PICC_IsNewCardPresent() && rfid.PICC_ReadCardSerial())

{

String uid = "";

for (byte i = 0; i < rfid.uid.size; i++)

{

if (rfid.uid.uidByte[i] < 0x10)

uid += "0";

uid += String(rfid.uid.uidByte[i], HEX);

}

uid.toUpperCase();

String msg = "RFID:" + uid;

sendToEsp(msg.c_str());

lcdShow("Dang kiem tra...");

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

vTaskDelay(pdMS_TO_TICKS(1000));

}

vTaskDelay(pdMS_TO_TICKS(100));

}

}

// ================= TASK: SERIAL COMMUNICATION =================

void taskSerial(void *pv)

{

(void)pv;

char buf[100];

while (1)

{

if (Serial1.available())

{

int idx = 0;

while (Serial1.available() && idx < 99)

{

char c = Serial1.read();

if (c == '\n')

break;

buf[idx++] = c;

}

buf[idx] = '\0';

String line = String(buf);

line.trim();

if (line.startsWith("ALLOW:"))

{

String userName = line.substring(6);

lastUserName = userName;

buzzSuccess();

lcdShow("Xin chao", (userName + "!").c_str());

vTaskDelay(pdMS_TO_TICKS(2000));

// Mở cửa

if (!doorOpen)

{

lcdShow("Cua dang mo...");

doorOpen = true;

autoCloseScheduled = false;

openDoor();

}

}

else if (line.indexOf("DENY") >= 0)

{

buzzError();

lcdShow("The khong", "hop le!");

vTaskDelay(pdMS_TO_TICKS(2000));

lcdShow("Moi quet the...");

}

else if (line.indexOf("CLOSE") >= 0)

{

lcdShow("Cua dang dong...");

doorOpen = false;

autoCloseScheduled = false;

closeDoor();

}

}

vTaskDelay(pdMS_TO_TICKS(20));

}

}

// ================= TASK: PIR SENSOR =================

void taskPIR(void *pv)

{

(void)pv;

TickType_t lastMotion = 0;

bool motionDetected = false;

while (1)

{

if (!systemReady)

{

vTaskDelay(pdMS_TO_TICKS(200));

continue;

}

if (digitalRead(PIR_PIN) == HIGH)

{

if (!motionDetected)

{

motionDetected = true;

lastMotion = xTaskGetTickCount();

sendToEsp("PIR:Motion");

// Mở cửa khi phát hiện chuyển động (giống RFID)

if (!doorOpen)

{

doorOpen = true;

autoCloseScheduled = false;

openDoor();

}

}

}

else

{

motionDetected = false;

}

vTaskDelay(pdMS_TO_TICKS(200));

}

}

// ================= TASK: AUTO CLOSE =================

void taskAutoClose(void *pv) {

(void)pv;

int lastSecondsLeft = -1;

while (1) {

// Kiểm tra nếu cần tự động đóng cửa

if (autoCloseScheduled && doorOpen) {

TickType_t elapsed = xTaskGetTickCount() - doorOpenedTime;

int secondsLeft = 5 - (elapsed / 1000);

// Chỉ cập nhật LCD khi số giây thay đổi

if (secondsLeft > 0 && secondsLeft <= 5 && secondsLeft != lastSecondsLeft) {

char line1[17];

char line2[17];

sprintf(line1, "Cua da mo!");

sprintf(line2, "%ds...", secondsLeft);

lcdShow(line1, line2);

lastSecondsLeft = secondsLeft;

}

// Đã đủ 5 giây, đóng cửa

if (elapsed >= pdMS_TO_TICKS(5000)) {

Serial3.println("[Auto Close] Closing door after 5 seconds");

doorOpen = false;

autoCloseScheduled = false;

lastSecondsLeft = -1;

closeDoor();

}

} else {

lastSecondsLeft = -1;

}

vTaskDelay(pdMS_TO_TICKS(100));

}

}

// ================= TASK: MOTOR CONTROL =================

void taskMotor(void *pv)

{

(void)pv;

MotorCmd cmd;

while (1)

{

if (xQueueReceive(xQueueMotor, &cmd, 0) == pdTRUE)

{

if (cmd == CMD_OPEN)

{

lcdShow("Cua dang mo...");

digitalWrite(MOTOR_IN1, HIGH);

digitalWrite(MOTOR_IN2, LOW);

analogWrite(MOTOR_ENA, 150);

motorRunning = true;

}

else if (cmd == CMD_CLOSE)

{

lcdShow("Cua dang dong...");

digitalWrite(MOTOR_IN1, LOW);

digitalWrite(MOTOR_IN2, HIGH);

analogWrite(MOTOR_ENA, 150);

motorRunning = true;

}

else

{

digitalWrite(MOTOR_IN1, LOW);

digitalWrite(MOTOR_IN2, LOW);

analogWrite(MOTOR_ENA, 0);

motorRunning = false;

}

}

// Check limit switches

if (motorRunning)

{

// Cửa đã mở xong

if (digitalRead(LIMIT_OPEN_PIN) == LOW && doorOpen)

{

analogWrite(MOTOR_ENA, 0);

digitalWrite(MOTOR_IN1, LOW);

digitalWrite(MOTOR_IN2, LOW);

motorRunning = false;

sendToEsp("Door:Opened");

lcdShow("Cua da mo!");

Serial3.println("[Motor] Door fully opened");

// Lên lịch tự động đóng sau 5 giây

doorOpenedTime = xTaskGetTickCount();

autoCloseScheduled = true;

Serial3.println("[Motor] Auto-close scheduled in 5 seconds");

vTaskDelay(pdMS_TO_TICKS(1000));

lcdShow("Moi quet the...");

}

// Cửa đã đóng xong

if (digitalRead(LIMIT_CLOSE_PIN) == LOW && !doorOpen)

{

analogWrite(MOTOR_ENA, 0);

digitalWrite(MOTOR_IN1, LOW);

digitalWrite(MOTOR_IN2, LOW);

motorRunning = false;

sendToEsp("Door:Closed");

lcdShow("Cua da dong!");

Serial3.println("[Motor] Door fully closed");

vTaskDelay(pdMS_TO_TICKS(1000));

lcdShow("Moi quet the...");

}

}

vTaskDelay(pdMS_TO_TICKS(50));

}

}

// ================= SETUP =================

void setup()

{

Serial.begin(115200);

Serial1.begin(115200);

Serial3.begin(9600);

SPI.begin();

rfid.PCD_Init();

Wire.begin();

lcd.init();

lcd.backlight();

pinMode(PIR_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(MOTOR_IN1, OUTPUT);

pinMode(MOTOR_IN2, OUTPUT);

pinMode(MOTOR_ENA, OUTPUT);

pinMode(LIMIT_OPEN_PIN, INPUT_PULLUP);

pinMode(LIMIT_CLOSE_PIN, INPUT_PULLUP);

xMutexSerial = xSemaphoreCreateMutex();

xMutexLCD = xSemaphoreCreateMutex();

xQueueMotor = xQueueCreate(4, sizeof(MotorCmd));

Serial3.println("=== STM32 SmartDoor System ===");

Serial3.println("Initializing...");

xTaskCreate(taskStartup, "Startup", 256, NULL, 4, NULL);

xTaskCreate(taskSerial, "Serial", 256, NULL, 3, NULL);

xTaskCreate(taskRFID, "RFID", 256, NULL, 2, NULL);

xTaskCreate(taskMotor, "Motor", 256, NULL, 2, NULL);

xTaskCreate(taskPIR, "PIR", 256, NULL, 1, NULL);

xTaskCreate(taskAutoClose, "AutoClose", 128, NULL, 1, NULL);

vTaskStartScheduler();

}

void loop()

{

// FreeRTOS handles everything

}

Code ESP32:

#include <Arduino.h>

#include <WiFi.h>

#include <WebSocketsClient.h>

#include <ArduinoJson.h>

// ===== WiFi Config =====

const char* WIFI_SSID = "NGUYEN TRUNG THANH";

const char* WIFI_PASS = "08082004";

// ===== Server Config =====

const char* SERVER_HOST = "192.168.0.100";  // IP of Python server

const int SERVER_PORT = 8000;

const char* DEVICE_TOKEN = "esp32-secret-token";

// ===== Serial to STM32 =====

#define STM32_RX 16  // ESP32 RX ← STM32 TX

#define STM32_TX 17  // ESP32 TX → STM32 RX

WebSocketsClient ws;

bool wsConnected = false;

unsigned long lastPingTime = 0;

unsigned long lastReconnectAttempt = 0;

const unsigned long PING_INTERVAL = 30000; // Ping every 30 seconds

const unsigned long RECONNECT_INTERVAL = 5000; // Try reconnect every 5 seconds

void sendToSTM32(const char* msg) {

Serial2.println(msg);

}

void webSocketEvent(WStype_t type, uint8_t* payload, size_t length) {

switch (type) {

case WStype_DISCONNECTED:

wsConnected = false;

Serial.println("[WS] Disconnected from server");

break;

case WStype_CONNECTED: {

wsConnected = true;

lastPingTime = millis();

Serial.printf("[WS] Connected to server: %s\n", payload);

// Send initial message to server

ws.sendTXT("{\"type\":\"device_connected\",\"device\":\"esp32\"}");

break;

}

case WStype_TEXT: {

Serial.printf("[WS] Received JSON: %s\n", payload);

// Parse JSON from server

StaticJsonDocument<256> doc;

DeserializationError error = deserializeJson(doc, payload);

if (error) {

Serial.printf("[WS] JSON parse error: %s\n", error.c_str());

return;

}

// Convert JSON to plain text for STM32

const char* action = doc["action"];

Serial.printf("[WS] Action received: %s\n", action);

if (strcmp(action, "open_door") == 0) {

const char* name = doc["name"] | "";

String msg = "ALLOW:" + String(name);

sendToSTM32(msg.c_str());

Serial.printf("[STM32] Sent: %s\n", msg.c_str());

}

else if (strcmp(action, "deny") == 0) {

sendToSTM32("DENY");

Serial.println("[STM32] Sent: DENY");

}

else if (strcmp(action, "close_door") == 0) {

sendToSTM32("CLOSE");

Serial.println("[STM32] Sent: CLOSE");

}

else if (strcmp(action, "system_locked") == 0) {

sendToSTM32("LOCKED");

Serial.println("[STM32] Sent: LOCKED");

}

else if (strcmp(action, "system_unlocked") == 0) {

sendToSTM32("UNLOCKED");

Serial.println("[STM32] Sent: UNLOCKED");

}

else {

Serial.printf("[WS] Unknown action: %s\n", action);

}

break;

}

case WStype_BIN:

Serial.printf("[WS] Binary data received, length: %u\n", length);

break;

case WStype_PING:

Serial.println("[WS] Ping received from server");

break;

case WStype_PONG:

Serial.println("[WS] Pong received from server");

break;

case WStype_ERROR:

Serial.printf("[WS] Error: %s\n", payload);

break;

default:

Serial.printf("[WS] Unknown event type: %d\n", type);

break;

}

}

void handleSTM32Serial() {

if (!Serial2.available()) return;

String line = Serial2.readStringUntil('\n');

line.trim();

if (line.length() == 0) return;

Serial.printf("[STM32] Received: %s\n", line.c_str());

// Convert STM32 plain text to JSON for server

if (wsConnected) {

StaticJsonDocument<256> doc;

if (line.startsWith("RFID:")) {

String uid = line.substring(5);

doc["event"] = "card_scanned";

doc["card_uid"] = uid;

}

else if (line.startsWith("PIR:")) {

doc["event"] = "motion_detected";

}

else if (line.startsWith("Door:Opened")) {

doc["event"] = "door_status";

doc["status"] = "opened";

}

else if (line.startsWith("Door:Closed")) {

doc["event"] = "door_status";

doc["status"] = "closed";

}

else {

// Unknown format, send as raw message

doc["event"] = "raw";

doc["message"] = line;

}

String jsonStr;

serializeJson(doc, jsonStr);

ws.sendTXT(jsonStr);

Serial.printf("[WS] Sent JSON: %s\n", jsonStr.c_str());

} else {

Serial.println("[WS] Not connected, message not sent");

}

}

void connectWiFi() {

if (WiFi.status() == WL_CONNECTED) {

return;

}

Serial.printf("\n[WiFi] Connecting to: %s\n", WIFI_SSID);

WiFi.mode(WIFI_STA);

WiFi.begin(WIFI_SSID, WIFI_PASS);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 30) {

delay(500);

Serial.print(".");

attempts++;

}

if (WiFi.status() == WL_CONNECTED) {

Serial.printf("\n[WiFi] Connected! IP: %s\n", WiFi.localIP().toString().c_str());

Serial.printf("[WiFi] Signal strength: %d dBm\n", WiFi.RSSI());

} else {

Serial.println("\n[WiFi] Connection failed!");

}

}

void connectWebSocket() {

Serial.println("[WS] Attempting to connect to WebSocket server...");

Serial.printf("[WS] Server: %s:%d\n", SERVER_HOST, SERVER_PORT);

String path = "/ws/device?token=" + String(DEVICE_TOKEN);

Serial.printf("[WS] Path: %s\n", path.c_str());

ws.begin(SERVER_HOST, SERVER_PORT, path.c_str());

ws.onEvent(webSocketEvent);

ws.setReconnectInterval(5000);

ws.enableHeartbeat(15000, 3000, 2);

lastReconnectAttempt = millis();

}

void setup() {

// Debug serial

Serial.begin(115200);

delay(1000);

Serial.println("\n\n=================================");

Serial.println("SmartDoor ESP32 Gateway");

Serial.println("=================================");

// Serial to STM32 (115200 baud to match STM32)

Serial2.begin(115200, SERIAL_8N1, STM32_RX, STM32_TX);

Serial.println("[Serial] STM32 communication initialized");

// Connect WiFi

connectWiFi();

// Connect WebSocket if WiFi is connected

if (WiFi.status() == WL_CONNECTED) {

connectWebSocket();

} else {

Serial.println("[ERROR] Cannot connect to WebSocket without WiFi");

}

Serial.println("=================================");

Serial.println("System ready!");

Serial.println("=================================\n");

}

void loop() {

// Handle WebSocket

ws.loop();

// Handle STM32 Serial

handleSTM32Serial();

// Send periodic ping to keep connection alive

if (wsConnected && (millis() - lastPingTime > PING_INTERVAL)) {

ws.sendPing();

lastPingTime = millis();

Serial.println("[WS] Sending ping to keep alive");

}

// Reconnect WiFi if disconnected

if (WiFi.status() != WL_CONNECTED) {

Serial.println("[WiFi] Connection lost! Reconnecting...");

connectWiFi();

// Reconnect WebSocket after WiFi is back

if (WiFi.status() == WL_CONNECTED && !wsConnected) {

connectWebSocket();

}

}

// Check WebSocket connection status periodically

if (!wsConnected && (millis() - lastReconnectAttempt > RECONNECT_INTERVAL)) {

if (WiFi.status() == WL_CONNECTED) {

Serial.println("[WS] Not connected. Attempting reconnect...");

lastReconnectAttempt = millis();

}

}

}